EXP NO:2(A)-GENERATION AND ANALYSIS OF FREQUENCY MODULATED

Objective 1: To generate a Frequency Modulated (FM) signal using MATLAB by varying the

frequency of a high-frequency carrier in accordance with the amplitude of the message signal

and to observe the resulting waveform in the time domain.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

Am=1;

Ac=1;

fm=15;

fc=60;

beta=5;

kp=10;

fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t); % Message signal

c = Ac*cos(2*pi*fc*t); % Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM

subplot(4,1,1), plot(t,m), title('Message Signal')

subplot(4,1,2), plot(t,c), title('Carrier Signal')

subplot(4,1,3), plot(t,fm_sig), title('FM Signal')

Result:

The Frequency Modulated (FM) signal was successfully generated using MATLAB, and

the time-domain waveforms of the message signal, carrier signal, and FM signal were observed and analyzed.

Objective 2: To analyze the frequency spectrum of the generated FM signal using FFT and

identify the carrier component, sideband frequencies, and the bandwidth of the FM signal based

on Carson's rule

PROGRAM (MATLAB CODE)

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signals

clc;

clear;

close all;

%% Input Parameters

Am = 2; % Message signal amplitude

fm = 500; % Message frequency (Hz)

Ac = 4; % Carrier amplitude

fc = 40000; % Carrier frequency (Hz)

kf = 1000; % Frequency sensitivity (Hz/V)

fs = 500000; % Sampling frequency (Hz)

T = 0.02; % Simulation duration (s)

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation Calculation

delta_f = kf * Am;

%% Modulation Index Calculation

beta = delta_f/fm;

%% FM Signal Generation

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Plot Message Signal

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Plot FM Signal

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1.5);

title('Frequency Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Spectrum Analysis

N = length(fm_signal);

FM_fft = fftshift(abs(fft(fm_signal))/N);

f = (-N/2:N/2-1)*(fs/N);

figure;

plot(f,FM_fft,'LineWidth',1.5);

title('FM Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

xlim([fc-10000 fc+10000]);

%% Display Results

fprintf('\nFM SIGNAL PARAMETERS\n');

fprintf('---------------------------\n');

fprintf('Carrier Frequency = %.0f Hz\n',fc);

fprintf('Message Frequency = %.0f Hz\n',fm);

fprintf('Message Amplitude = %.2f V\n',Am);

fprintf('Frequency Sensitivity = %.0f Hz/V\n',kf);

fprintf('\nCALCULATED VALUES\n');

fprintf('---------------------------\n');

fprintf('Frequency Deviation (Δf) = %.0f Hz\n',delta_f);

fprintf('Modulation Index (β) = %.2f\n',beta);

%% Carson's Rule Bandwidth

BW = 2*(delta_f + fm);

fprintf('Bandwidth (Carson Rule) = %.0f Hz\n',BW);

OUTPUT:

FM SIGNAL PARAMETERS
---------------------------
Carrier Frequency = 40000 Hz
Message Frequency = 500 Hz
Message Amplitude = 4.00 V
Frequency Sensitivity = 1000 Hz/V

CALCULATED VALUES
---------------------------
Frequency Deviation (Δf) = 4000 Hz

Modulation Index (β) = 8.00
Bandwidth (Carson Rule) = 9000 Hz

>>

Result

i) The time-domain waveform was observed, and the frequency spectrum obtained using FFT has

been plotted.

ii) Also, calculated the frequency deviation and bandwidth using Carson’s rule.

EXP-2(B)  CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS

Objectives:

To generate a Frequency Modulated (FM) signal in MATLAB using specified message and

carrier signal parameters, and to determine the frequency deviation and modulation index of the

FM signal while analyzing the relationship between frequency deviation, message frequency,

and modulation index.

PROGRAM (MATLAB CODE)

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signal

clc;

clear;

close all;

%% Message Signal Parameters

Am = 2; % Message amplitude (V)

fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters

Ac = 18; % Carrier amplitude (V)

fc = 20000; % Carrier frequency (Hz)

%% FM Parameters

kf = 250; % Frequency sensitivity (Hz/V)

fs = 100000; % Sampling frequency (Hz)

T = 0.01; % Simulation duration (s)

%% Time Vector

t = 0:1/fs:T;

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation

delta_f = kf*Am;

%% Modulation Index

beta = delta_f/fm;

%% FM Signal

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters

fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);

fprintf('Message Frequency (fm) = %.0f Hz\n',fm);

fprintf('Message Amplitude (Am) = %.0f V\n',Am);

fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);

fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);

fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification

beta_verify = delta_f/fm;

fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check

tolerance = 1e-6;

if abs(beta - beta_verify) < tolerance

fprintf('\nVerification Successful\n');

else

fprintf('\nVerification Failed\n');

end

%% Plotting

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

grid on;

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1);

grid on;

title('FM Signal');

xlabel('Time (s)');

ylabel('Amplitude (V)');

OUTPUT:

Carrier Frequency (fc) = 20000 Hz
Message Frequency (fm) = 500 Hz
Message Amplitude (Am) = 2 V
Frequency Sensitivity (kf) = 250 Hz/V
Frequency Deviation (Delta f) = 500.00 Hz
Modulation Index (Beta) = 1.00
Verified Modulation Index = 1.00
Verification Successful

RESULT:

This experiment verifies that the modulation index is equal to the ratio of frequency deviation to message frequency, i.e., β=Δf/f_m.